IP Addressing Plan

Ubuntu Innovations (Pty) Ltd

Week 2 – Task 5  |  IT Support Capstone Project

1. Network Overview

Ubuntu Innovations (Pty) Ltd operates a segmented office network using the private IP range 192.168.0.0/16. The network is divided into five VLANs, each assigned to a specific device category. This segmentation improves security, simplifies traffic management, and allows department-level access control.

The router/firewall serves as the central gateway at 192.168.1.1 and manages all inter-VLAN routing and internet access.

2. Subnet Allocation Table

The following subnets have been assigned to support all 25 employees and network devices across five departments:

| Device Type | Subnet | IP Range | Gateway | Usable Hosts |
| --- | --- | --- | --- | --- |
| Servers / NAS | 192.168.10.0/24 | 192.168.10.1 – 192.168.10.254 | 192.168.10.1 | 253 |
| User Devices | 192.168.20.0/24 | 192.168.20.1 – 192.168.20.254 | 192.168.20.1 | 253 |
| Printers | 192.168.30.0/24 | 192.168.30.1 – 192.168.30.254 | 192.168.30.1 | 253 |
| Wi-Fi (Wireless) | 192.168.40.0/24 | 192.168.40.1 – 192.168.40.254 | 192.168.40.1 | 253 |
| Management / Admin | 192.168.50.0/24 | 192.168.50.1 – 192.168.50.254 | 192.168.50.1 | 253 |

3. Static IP Assignments

The following infrastructure devices have been assigned static IP addresses to ensure consistent network identity and reliable connectivity:

| Device | IP Address | Notes |
| --- | --- | --- |
| Router / Firewall | 192.168.1.1 | Main gateway for all traffic |
| Managed Switch | 192.168.1.2 | Admin management interface |
| WAP 1 | 192.168.1.3 | Wireless Access Point – Zone A |
| WAP 2 | 192.168.1.4 | Wireless Access Point – Zone B |
| NAS Device | 192.168.10.10 | File storage and backups |
| Printer 1 | 192.168.30.10 | Shared laser printer |
| Printer 2 | 192.168.30.11 | Shared laser printer |

4. DHCP Configuration

User desktops, laptops, and wireless devices receive IP addresses dynamically via DHCP. A portion of each subnet is reserved for future static assignments or infrastructure expansion.

| Subnet | DHCP Range | Excluded (Static Reserved) |
| --- | --- | --- |
| User Devices | 192.168.20.50 – 192.168.20.200 | .1 – .49 reserved for static devices |
| Wi-Fi | 192.168.40.50 – 192.168.40.200 | .1 – .49 reserved for static devices |

5. DNS Configuration

Internal DNS resolution is handled by the NAS/server at 192.168.10.10. External DNS queries are forwarded to the following public resolvers for redundancy:

- Primary: 8.8.8.8 (Google Public DNS)

- Secondary: 1.1.1.1 (Cloudflare DNS)

6. Design Justification

The use of separate subnets for each device category aligns with IT security best practices. Key benefits include:

- Isolation: Printer and server traffic is separated from general user traffic, reducing exposure to lateral movement attacks.

- Scalability: Each /24 subnet supports up to 253 hosts, providing ample room for future growth.

- Manageability: Static IPs on infrastructure devices ensure reliable remote management and monitoring.

- Policy enforcement: VLAN segmentation enables firewall rules to restrict inter-subnet access where required.